Name: - khan naseer 	Sem: - III

Roll no: - 25BIT34	Sub: - Computer Network

Class: - SY-BSCIT	Batch:-2

# 🖥️ DHCP

(Dynamic Host Configuration Protocol)

Configuring DHCP Server In Packet Tracer

DHCP (Dynamic Host Configuration Protocol) is a network management protocol that automatically assigns an IP address and other network settings to devices when they connect to a network

STEP 1:- OPEN PACKET TRACER

STEP 2:- AND FOR DHCP TO WORK, WE NEED PC’S A SERVER, A SWITCH AND A ROUTER

STEP 3:- AS SHOWN IN THE DEMONSTRATION, IT CONTAINS 3 SETS OF 2 PC’S AND THREE SERVERS AND THREE SWITCHES WITH ONE ROUTER

STEP 4:- CONNECT THE DEVICES BY USING CONNECTION CABLE THEN FIRST CONNECT THE PC’S AND SERVER TO THE SWITCH, THEN CONNECT THE SWITCH TO ROUTER, DO THE SAME FOR THE TWO OTHERS CONNECTIONS

STEP 5:-  THERE IS NO CONNECTION GIVEN IN THE ROUTER WHILE TRYING TO CONNECT THE THIRD SWITCH TO THE ROUTER > CLICK > ROUTER > SWITCH OFF THE ROUTER (GREEN INDICATION SWITCH) > SELECT > NM-2FE2W > DRAG THE TWO EXTRA PORT AND ATTACH IT TO THE EMPTY SLOT OF THE ROUTER > SWITCH IT ON (AFTER THAT WAIT FOR A MINUTE BEFORE CONNECTING THIRD SWITCH TO THE ROUTER)

STEP 6:- SELECT THE CABLE FROM THE THIRD SWITCH AND ATTACH IT TO THE ROUTER

STEP 7:- FIRST SERVER CONFIGURE > DESKTOP > IP CONFIGURE > IPV4-192.168.0.2 > SUBNET MASK > DEFAULT GATEWAY-192.168.0.1 > DNS SERVER-192.168.0.2

STEP 8:- SECOND SERVER CONFIGURE > DESKTOP > IP CONFIGURE > IPV4-192.168.1.2 > SUBNET MASK > DEFAULT GATEWAY-192.168.1.1 > DNS SERVER-192.168.1.2

STEP 9:- THIRD SERVER CONFIGURE > DESKTOP > IP CONFIGURE > IPV4-192.168.2.2 > SUBNET MASK > DEFAULT GATEWAY-192.168.2.1 > DNS SERVER-192.168.2.2

STEP 10:- ROUTER > CONFIG > INTERFACE > FASTETHERNET0/0 > IPV4-192.168.0.1 > SUBNET MASK > FASTETHERNET0/1 > IPV4-192.168.1.1 SUBNET MASK FASTETHERNET1/0 > IPV4-192.168.2.1 > SUBNET MASK

STEP 11:- ROUTER > CLI > (WRITE) INTERFACE FASTETHERNET0/0 (ENTER)IP HELPER-ADDRESS 192.168.0.2 (YOU HAVE CONFIGURED THE IP ADDRESS TO THE ROUTER) DO THE SAME FOR THE OTHER TWO CABLE (FASTETHERNET0/1 TO FASTETHERNET1/0) THEN CLICK ENTER & WRITE :exit >

STEP 12:- GO TO SERVER0 > SERVICES > DHCP > SERVICE-ON > DEFAULT GATEWAY-192.168.0.1 > DNS SERVER-192.168.0.2 > SAVE > POOL NAME-SERVERPOOL (AFTER THAT CREATE TWO MORE SERVERPOOL CREATE IN THE SAME SERVER)

NAME-SERVERPOOL1 > SERVICE-ON > DEFAULT GATEWAY-192.168.1.1 > DNS SERVER-192.168.0.2 > START IP ADDRESS- 192.168.1.0 > ADD > SAVE

NAME-SERVERPOOL2  > SERVICE-ON > DEFAULT GATEWAY-192.168.2.1 > DNS SERVER-192.168.0.2 > START IP ADDRESS- 192.168.2.0 > ADD > SAVE

STEP 13:- GO ON ANY PC > IP CONFIGURATION > CLICK ON DHCP