Name:- Ansari Mohd Ayan Nasiruddin	Branch:SYBscIT

RollNo:-25BIT41 	Sem:-III

Subject:- Computer Network	Batch:- 2

Practical3

| 3.Configuring LAN setup |  |
| --- | --- |
| a | Planning and Setting IP networks. |
| b | Basic network troubleshooting. |

Aim: a) Planning and Setting IP networks.

b) Basic network troubleshooting.

- Planning and Setting IP networks.

1. Understand the Requirements

- Identify the purpose of the network (home, office, data center, etc.).

- Estimate the number of devices (PCs, phones, printers, servers, etc.).

- Plan for future expansion (scalability).

### 2. Choose the Network Type

- LAN (Local Area Network): For small, local setups like offices or homes.

- WAN (Wide Area Network): For connecting across cities or countries.

- MAN (Metropolitan Area Network): For large campuses or towns.

### Select IP Addressing Scheme

#### 🔹 a) IP Version

- IPv4: Most common (e.g., 192.168.1.1)

- IPv6: Newer, for larger addressing space (e.g., 2001:0db8::1)

#### 🔹 b) Public vs Private IP

- Private IPs (for internal use):

  - 10.0.0.0 – 10.255.255.255

  - 172.16.0.0 – 172.31.255.255

  - 192.168.0.0 – 192.168.255.255

- Public IPs (for internet-facing devices): Assigned by ISP

#### c) Static vs Dynamic IP

- Static IP: Manually assigned, doesn’t change (used for servers).

- Dynamic IP: Assigned automatically by DHCP, changes over time.

### 4. Subnetting

- Divide IP addresses into smaller networks (subnets) for better management and security.

- Helps in efficient IP address usage.

- Example: From 192.168.1.0/24 to multiple subnets like /25, /26.

### 5. Plan Network Devices

- Router: Connects LAN to the internet.

- Switch: Connects devices within the same LAN.

- Access Points: For wireless devices (Wi-Fi).

- Firewall: Provides security by controlling traffic.

### 6. Assign IP Addresses

- Reserve IPs for:

  - Router/Gateway (e.g., 192.168.1.1)

  - Servers/Printers

  - DHCP Pool (for client devices)

- Ensure no IP conflicts (no two devices have the same IP).

### 7. Configure DHCP (if needed)

- Automatically assigns IP addresses to client devices.

- Set range (e.g., 192.168.1.100 to 192.168.1.200).

### 8. Test the Network

- Use ping command to test connectivity.

- Use ipconfig/ifconfig to check IP configurations.

- Check internet access and internal communication.

### 9. Document the Network Plan

- Record IP ranges, device roles, MAC addresses, static IPs.

- Helps in troubleshooting and future upgrades.

### 10. Monitor and Maintain

- Use tools like Wireshark, PRTG, or Nagios for monitoring.

- Regularly update firmware and security settings.

- Review and expand IP plan as the network grows.

- Basic network troubleshooting.

Network troubleshooting is the systematic process of searching for, diagnosing, and correcting network issues.

### 1. Check Physical Connections

- Ensure all cables (Ethernet, power) are properly connected.

- Look for loose or damaged cables.

- Check indicator lights on modem/router/switch.

### 2. Restart Devices

- Restart the computer, router, and modem.

- Power cycling often fixes temporary issues.

### 3. Check IP Configuration

- On Windows:
Open Command Prompt → type ipconfig

- On Linux/macOS:
Use ifconfig or ip a

- Check if the device has a valid IP (not 169.x.x.x).

### 4. Use Ping Command

- Test connection to:

  - Local router: ping 192.168.1.1

  - Internet: ping 8.8.8.8 (Google DNS)

  - Website: ping google.com

- No reply = connection issue.

### 5. Check DNS Settings

- If pinging IP works but website doesn’t, it's a DNS issue.

- Try using public DNS:

  - Google DNS: 8.8.8.8, 8.8.4.4

  - Cloudflare DNS: 1.1.1.1

### 6.Check Firewall or Antivirus

- Sometimes, they block network access.

- Temporarily disable to test (then re-enable).

### 7.Contact ISP

- If everything seems fine but internet still doesn’t work,
contact your Internet Service Provider.