Name:- Ansari Mohd Ayan Nasiruddin	Branch:SYBscIT

RollNo:-25BIT41 	Sem:-III

Subject:- Computer Network	Batch:- 2

Practical4

| 4.TCP/IP Configuration and Network Resource Sharing in Windows/Linux. |  |
| --- | --- |
| a | Configuration of TCP/IP Protocols in Windows / Linux. |
| b | Implementation of Drive/file sharing and printer sharing. |

Aim: a) Configuration of TCP/IP Protocols in Windows / Linux.

b) Implementation of Drive/file sharing and printer sharing.

a) Configuration of TCP/IP Protocols in Windows / Linux.

## Configuration of TCP/IP in Windows

### 1. Open Network Settings

- Press Windows + R, type ncpa.cpl, and press Enter.
 (This opens the Network Connections window.)

### 2. Select the Network Adapter

- Right-click on the active network adapter (e.g., Ethernet or Wi-Fi).

- Click Properties.

### 3. Choose TCP/IPv4

- In the Networking tab, select Internet Protocol Version 4 (TCP/IPv4).

- Click Properties.

### 4. Configure IP Settings

- For Static IP:

  - Select Use the following IP address

  - Enter:

    - IP address: e.g., 192.168.1.10

    - Subnet mask: e.g., 255.255.255.0

    - Default gateway: e.g., 192.168.1.1

  - (Optional) Configure Preferred DNS and Alternate DNS.

- For Dynamic IP (DHCP):

  - Select Obtain an IP address automatically

  - Select Obtain DNS server address automatically

### 5. Save and Verify

- Click OK, then close the windows.

- Open Command Prompt and type:

### Linux .Permanent IP Configuration (GUI)

- Go to Settings → Network.

- Select the active adapter (e.g., Wired).

- Click the gear icon .

- Under IPv4, choose Manual.

- Enter:

  - IP address: 192.168.1.20

  - Netmask: 255.255.255.0

  - Gateway: 192.168.1.1

Click Apply and reconnect.

b) Implementation of Drive/file sharing and printer sharing.

## 1. Drive / File Sharing in Windows

### Step 1: Select the Folder to Share

- Right-click the folder you want to share.

- Select Properties → Sharing tab.

- Click Advanced Sharing… → check Share this folder.

- Optionally, click Permissions to set:

  - Read → users can only view files.

  - Change → users can modify files.

  - Full Control → users can modify and manage permissions.

- Click OK to save.

### Step 2: Access the Shared Folder

- From another computer on the same network:

  - Press Windows + R, type \\ComputerName\SharedFolderName → Enter.

  - Or open File Explorer → Network and select the shared computer.

## 2. Printer Sharing in Windows

### Step 1: Share the Printer

- Go to Control Panel → Devices and Printers.

- Right-click the printer → Printer properties → Sharing tab.

- Check Share this printer and give it a share name.

- Click Apply → OK.

### Step 2: Connect to Shared Printer on Another Computer

- Option 1: Using Network Path

  - Press Windows + R, type \\ComputerName\PrinterShareName → Enter.

  - Follow prompts to install drivers if needed.

- Option 2: Using “Add a Printer”

  - Go to Control Panel → Devices and Printers → Add a Printer.

  - Select The printer that I want isn’t listed → Select a shared printer by name.

  - Enter \\ComputerName\PrinterShareName → Next → Finish.

## Important Notes

- All computers must be on the same network/workgroup.

- Ensure File and Printer Sharing is enabled in Control Panel → Network and Sharing Center → Advanced Sharing Settings.

- Proper permissions are essential to control access to files and printers.

- Firewalls may block sharing; ensure exceptions are enabled for file and printer sharing.