Name: - khan naseer 	Sem: - III

Roll no: - 25BIT34	Sub: - Computer Network (CN)

Class: - SY-BSCIT	Batch:-2

===========================================================================

Exp5(a) Given an IP address 165.1.2.1 and network mask, determine other information about the IP address such as.

1)  Find Network Address.

2)  Find Network Broadcast Address.

3) Find the Total Number of Host Bits.

4)  Find Number of hosts?

5) Calculate the Number of Valid Hosts per Subnet

Classful Addressing – IP Address: 165.1.2.1

1) Find Network Address IP Address: 165.1.2.1 Subnet Mask: 255.255.0.0 Network Address = 165.1.0.0

### 2) Find Network Broadcast Address For the broadcast address, all host bits become 1: Network: 165.1.0.0 Network Broadcast: 165.1.255.255

### 3) Find Total Number of Host Bits Class B default mask is /16. Total IPv4 Bits = 32 Network Bits = 16 (Total Host Bits = 16 ) (Host Bits = 32 − 16 = 16 (H-N) )

4) Find Number of hosts? 2h (h host bits) 2¹⁶ = 65,536

4) Calculate theNumber of Valid Hosts per Subnet Using the formula shown in your image: ## Number of Hosts = 2ʰ − 2 —--------------------------------------------------------------------------------------- h = 16

Number of Valid Hosts = 2¹⁶ − 2 = 65,536 − 2 = 65,534 Calculate the Number of Valid Hosts per Subnet = 65,534

Exp5 (b)

Given an IP address   140.34.12.64/26  and a custom subnet mask, determine detailed subnetting information .

1) Subnet Address of this IP.

2) Broadcast Address of this subnet.

3) Range of Host Addresses (from first to last valid host).

4) Maximum Number of Subnets (within the original class).

5) Number of Hosts per Subnet.

6) Number of Subnet Bits.

Given IP Address: 140.34.12.64/26

Class: B
 Default Class B Mask: /16 = 255.255.0.0
 Given Subnet Mask: /26 = 255.255.255.192

### 1) Find Subnet Address of this IP Block Size = 256 − 192 = 64 Subnet ranges: Subnet; Address Range. 1; 140.34.12.0 – 140.34.12.63. 2; 140.34.12.64 – 140.34.12.127. 3; 140.34.12.128 – 140.34.12.191. 4; 140.34.12.192 – 140.34.12.255. Subnet Address = 140.34.12.64

### 2) Find Broadcast Address of This Subnet ### The given subnet starts at: ### 140.34.12.64 ### The next subnet starts at: ### 140.34.12.128 ### Therefore, the address just before the next subnet is the broadcast address. ### Broadcast Address = 140.34.12.127

### 3) Find Range of Host Addresses Subnet Address = 140.34.12.64 Broadcast Address = 140.34.12.127 The first address and last address cannot be assigned to hosts. First Valid Host = 140.34.12.65 Last Valid Host = 140.34.12.126 Therefore: Host Address Range = 140.34.12.65 – 140.34.12.126

### 4) Maximum Number of Subnets Class B default network bits = 16 Given subnet mask = /26 Subnet Bits = 26 − 16 = 10 bits Formula: Number of Subnets = 2ⁿ Where n = 10 Number of Subnets = 2¹⁰ = 1,024 Maximum Number of Subnets = 1,024

### 5) Number of Hosts per Subnet Total IPv4 bits = 32 Network bits = 26 Host Bits = 32 − 26 = 6 Formula: Number of Hosts = 2ʰ = 2⁶ = 64 Therefore: Total Addresses per Subnet = 64 Valid Hosts per Subnet = 2⁶ − 2 = 62

### 6) Number of Subnet Bits Class B default network bits = 16 Given network bits = 26 Subnet Bits = 26 − 16 Subnet Bits = 10 bits.